Energy M&V Baseline Optimizer

Sigma Computing — Process Guide

How to upload data, run the analysis, and use the interactive workbook

# 1. Overview

The M&V Sigma workbook performs the same IPMVP Option C baseline optimization analysis as the VS Code tool (see other documentation) but runs entirely in your browser through Sigma Computing. You upload/replace your CSV file referencing the provided template, click Run at the top of Page 2, and the results appear as interactive tables and charts.

The workbook has three pages:

| Page | Name | What it does |
| --- | --- | --- |
| Page 1 | Data Intake | Upload your CSV file here. The workbook reads it and passes it to the analysis engine. |
| Page 2 | Optimization & Models | The Python engine runs here. Shows 7 output tables plus quality scatter charts. |
| Page 3 | Savings & Cost Analysis | KPI tiles, savings charts, cumulative trend, and outlier summary — interactive with site filter. |

# 2. Preparing Your CSV File

## 2.1 Use the template

Download the blank templet at the top of Page 1 and use it as your starting point. This ensures all the data is in the appropriate order.

## 2.2 Required columns

The CSV must have exactly these column names. They are case-sensitive — do not rename them.

| Column Name | Required | Notes |
| --- | --- | --- |
| Site Name | Yes | Unique building identifier. All rows for the same building must use the same value. |
| Meter Number | No | Optional. Utility account or meter ID. If blank, the engine treats the whole site as a single meter. |
| Service Type | Yes | Must contain 'Electricity' or 'Natural Gas'. Case-insensitive — 'electric', 'gas' also work. |
| Period Start Date | Yes | Format: YYYY-MM-DD (e.g. 2023-01-01) |
| Period End Date | Yes | Format: YYYY-MM-DD |
| HDD | Yes | Heating Degree Days for the billing period. |
| CDD | Yes | Cooling Degree Days for the billing period. |
| Consumption | Yes | Energy usage. Standard units: kWh for electricity, Therms for gas. |
| Total Charges | Yes | Total dollar cost of the bill. Can include a $ sign and commas — the engine strips them automatically. |
| Commission Date | Yes | Date the energy efficiency project was commissioned. All rows for the same site use the same date. Bills before this date = baseline. Bills on/after = reporting. |
| Consumption Unit | No | Leave blank for kWh/Therms. Fill in for MWh, CCF, MCF, or MMBTU — the engine converts automatically. |
| Selected Baseline | No | Leave blank on first upload. After the first run, download the input_with_baseline output table as CSV — this file has “1” populated in this column for the selected baseline bills. Re-upload that file to run savings calculations without re-optimizing. See Section 4 for the full re-upload workflow. |

## 2.3 Data requirements

- Minimum 12 electricity billing rows before Commission Date per meter

- Minimum 8 gas billing rows before Commission Date per meter

- At least 1 billing row per calendar quarter in the baseline period

- All sites can be in the same CSV file — the engine groups by Site Name automatically

- For multi-meter sites, include all meters in the CSV — the engine detects whether they are concurrent or sequential

## 2.4 What to watch out for

- Total Charges formatted as currency (e.g. $1,234) is handled — the engine strips $ and commas

- Commission Date with time zone information (e.g. 2024-01-15 00:00:00+00:00) is handled — time zone is stripped automatically

- Commission Date must be present for every row — do not leave it blank even for reporting period rows

- Site Name must match exactly across rows — 'SiteA' and 'Site_A' will be treated as different sites

# 3. Uploading Your Data (Page 1)

Step 1    Upload your CSV

Click the CSV Input Table element on the page. Select Upload CSV in the drop down and proceed with your upload.

* Note: whatever data is currently loaded will be overwritten.

Step 2    Verify the data loaded correctly

After uploading, the table should show your data rows with the correct column headers. Quick checks:

- Row count matches your CSV (minus the header row)

- Site Name, Meter Number, and Commission Date columns are populated

- Period Start Date and Period End Date look like dates, not text

- Consumption and Total Charges look like numbers (dollar signs and commas are fine at this stage)

# 4. Running the Analysis (Page 2)

Step 1    Click Run

The engine will run. Expect:

- Small portfolios (1–5 sites): 5–15 seconds

- Medium portfolios (10–20 sites): 30–60 seconds

- Large portfolios (20+ sites): 60–120 seconds

Step 2   Review the 7 output tables

The Python element produces 7 output tables. Each is a child element wired to the Python output:

| Table | What it shows |
| --- | --- |
| individual_models | One row per meter. Your primary results — R², CV-RMSE, savings, weight flags. |
| aggregated_models | One row per concurrent multi-meter site. Only populated if your data has multi-meter sites with overlapping billing periods. |
| baseline_bills | One row per baseline bill per meter. Status column shows Used in Model, or Excluded with reason. |
| reporting_bills | One row per reporting bill. Includes Predicted Consumption, Avoided Usage, Pct Avoided, and Bill Avoided Cost. |
| outliers | Bills removed from baseline or excluded from savings. Shows Outlier_Type and Outlier_Reason. |
| meter_weights | Load distribution per meter for concurrent multi-meter sites. Shows Meter Weight (%), Coverage Flag, and Aggregate Weight Flag. |
| input_with_baseline | Re-upload file. Your original input CSV with one extra column: Selected Baseline = “1” for each bill the optimizer chose. Download this after the first run and use it for all future uploads. See Section 4 — Re-upload Workflow. |

## 4.1 Re-upload Workflow (adding new reporting bills)

After the first run, the baseline is locked. You do not need to re-optimize when new reporting bills come in — the engine detects the locked baseline automatically and skips optimization entirely.

- After the first run, download the input_with_baseline table as a CSV file.

- Open that CSV. It has the same columns as your original upload plus a Selected Baseline column with “1” marking the chosen baseline bills.

- Append your new reporting bill rows at the bottom of that CSV. Leave the Selected Baseline column blank for the new rows.

- Upload that CSV to the Input table on Page 1, replacing the previous input. Run the engine again on Page 2.

- The Optimization Mode column on individual_models will show “Locked” confirming the baseline was not changed. Savings results update to include the new reporting bills.

To re-optimize from scratch (e.g. for a rebaseline), start with a fresh CSV that has no Selected Baseline column and run as normal.

# 5. Using the Savings Dashboard (Page 3)

Page 3 is the interactive results dashboard. With no site selected, everything shows portfolio-level totals, but the results can be filtered down by site and service type for further investigation.

## 5.1 Filter controls

At the top of Page 3 there are five filter controls:

- Site Name (drop down) – select one or more sites to filter all elements on the page. Leave blank for the full portfolio view.

- Service Type (drop down) – filter to Electricity only, Natural Gas only, or both.

- Savings Status (drop down) – filter to Included or Excluded

- Model Passes Quality (drop down) – select true or false

- % Avoided Usage (slider) – adjust range to view sites with more or less % avoided usage

## 5.2 KPI tiles

## All are shown at a comprehensive portfolio level as well as broken down by fuel type.

| Tile | What it shows |
| --- | --- |
| Site Count | Total sites included in savings calculations |
| Total Avoided Cost ($) | Sum of avoided cost across all passing models. With a site selected: that site's total savings. |
| Avg % Avoided Usage | Average percent of predicted consumption that was avoided. Positive = savings. Negative = net usage increase. |
| Avg Monthly Avoided Cost | Average avoided cost per reporting bill — the expected ongoing monthly savings rate. |

## 5.3 Charts

| Chart | How to read it |
| --- | --- |
| Avoided Cost by Site and Fuel Type | Horizontal bars — length = total avoided cost. Color = service type. Negative bars = net usage increase. Sorted highest to lowest. |
| Avg % Avoided Usage by Site and Fuel Type | Same layout as avoided cost but normalized by size. |
| R² Distribution | Histogram showing how model quality is distributed across the portfolio. Most models should cluster to the right (high R²). Models near the threshold line are borderline and worth monitoring. |
| Savings Performance Scatter | Each dot = one model. X axis = % avoided, Y axis = avoided cost. Top-right = high savings in both dimensions. Bottom-left = underperforming. Negative X = using more than baseline. |
| Actual vs Predicted Consumption | Two lines per site/service type: actual consumption (what the meter measured) vs predicted consumption (what the baseline model says it should be). The gap between the lines is the avoided usage. Select a site and service type for the clearest view. |

# 6. Output Column Reference

## 6.1 individual_models and aggregated_models

| Column | Description |
| --- | --- |
| Site Name | Site identifier from input data |
| Meter Number | Meter identifier. 'AGGREGATED (N)' for aggregated rows. |
| Model Type | Individual or Aggregated |
| Service Type | Electricity or Natural Gas |
| Seasonal Strategy | Optimal = at least 2 bills per quarter selected. Acceptable = at least 1 per quarter (fallback used). |
| Baseline Count | Number of bills used to build the model |
| Reporting Count | Number of clean reporting bills used in savings calculation |
| Model Equation | Regression equation: Consumption = Intercept + HDD*coef + CDD*coef |
| R2 | R² score (0–1). Higher is better. Threshold: 0.70 electricity, 0.40 gas. |
| CV-RMSE | Average prediction error as % of mean consumption. Lower is better. Threshold: ≤15%. |
| Weather Coverage (%) | % of weather range covered by selected baseline bills. Threshold: ≥75%. |
| Model Passes Quality | True if R², CV-RMSE, and Weather Coverage all pass. Primary quality indicator. |
| Pct Avoided Usage | % of predicted consumption avoided in reporting period. Positive = savings. |
| Avoided Cost ($) | Dollar value: Modeled Cost minus Actual Cost |
| Avg Monthly Avoided Cost | Avoided Cost divided by number of reporting bills |
| Meter Weight (%) | This meter's share of total baseline consumption. Only populated for concurrent multi-meter sites. |
| Coverage Flag | LOW COVERAGE if a passing meter represents less than 25% of site baseline load |
| Weight Flag | OK / REVIEW / HIGH RISK based on failing meter load share (see Section 7.3) |
| Notes | Auto-generated observations: multi-meter quality comparisons, weight warnings, sequential meter notes |

## 6.2 reporting_bills

| Column | Description |
| --- | --- |
| Status | Used in Savings (included in calculations) or Excluded: reason (outlier removed) |
| Consumption | Actual metered consumption for this billing period |
| Predicted Consumption | What the baseline model predicted for this period given actual weather |
| Avoided Usage | Predicted minus Actual consumption |
| Pct Avoided | Avoided Usage / Predicted * 100% |
| Bill Avoided Cost | Bill-level avoided cost = Avoided Usage × measured rate for this bill. Used for the cumulative savings chart. |

## 6.3 Weight flag thresholds

| Flag | Condition and meaning |
| --- | --- |
| OK | Failing meters represent ≤33% of site baseline load. Aggregate is defensible for M&V reporting. |
| REVIEW | Failing meters represent 33–50% of load. A meaningful portion of site consumption is from unverified meters. Review before including in formal reports. |
| HIGH RISK | Failing meters represent >50% of load. More than half of site consumption cannot be individually verified. Consider excluding from savings totals or flagging explicitly. |
| LOW COVERAGE | On individual model rows: this passing meter represents less than 25% of site baseline load. Its contribution to the aggregate savings figure is small. |

# 7. Updating the Engine Script

When a new version of sigma_mv_engine.py is released, update the workbook by replacing the Python element's code. The element can be found in the hidden sheet labeled “Code”

## 7.1 Tunable parameters

These constants are near the top of the engine script and can be adjusted without affecting anything else:

| Parameter | Default | Effect |
| --- | --- | --- |
| MAX_COMBINATIONS | 500 | Max baseline combinations tested per meter. Higher = better optimization, slower runtime. 500 gives ~60s for 20 sites. |
| ELEC_R2_THRESHOLD | 0.70 | Minimum R² for electricity models to pass. |
| GAS_R2_THRESHOLD | 0.40 | Minimum R² for gas models to pass. |
| CVRMSE_THRESHOLD | 15.0 | Maximum CV-RMSE (%) to pass. ASHRAE Guideline 14. |
| WEIGHT_REVIEW_THRESHOLD | 0.33 | Failing meter weight above which aggregate gets REVIEW flag. |
| WEIGHT_HIGH_RISK_THRESHOLD | 0.50 | Failing meter weight above which aggregate gets HIGH RISK flag. |
| LOW_COVERAGE_THRESHOLD | 0.25 | Meter weight below which a passing individual meter gets LOW COVERAGE flag. |